SELENITE PROGRAMME

Robotic Fleet Engineering Specifications

v9.0 — All ECNs Applied (ECN-012 through ECN-019 Rev C)

200-Year Programme Architecture — 2.5 Mt/yr REO Committed Target

REF: SEL_ROBOT_FLEET v8 + ECN-012/013/014/015/016/017/019 Rev C. Lead: Jason (Systems Engineering). Status: DRAFT. April 2026.

This document supersedes SEL_ROBOT_FLEET v8 and all ECN amendments (ECN-012 through ECN-017). It incorporates ECN-019 Rev C programme-wide design revision. Individual robot design specifications remain in their standalone documents (SEL_MOLEI_DESIGN_v5, SEL_PROBE_DESIGN_RevA, SEL-HAULER-001_RevB, etc.) — this document provides the fleet-level architecture, scaling, and cross-references.

# 1. Fleet Overview

The Selenite robotic fleet comprises twelve vehicle classes. SPA vehicles are Earth-manufactured throughout. PKT vehicles transition to ~57% in-situ manufacture from P10. All fleet counts are demand-driven from REO production targets.

| Class | Type | Primary Role | Base | Initial | Peak |
| --- | --- | --- | --- | --- | --- |
| PROBE Mk I | Spacecraft | Round-trip Ni-Fe/PGM return (chemical) | SPA | 3 | 96 (P8) |
| PROBE Mk II | Spacecraft | Heavy asteroid processor, in-situ beneficiation | SPA | — | 260 (P11) |
| PROBE Mk III | Spacecraft | Ion SEP, never lands, EM anchoring, bulk mining | Orbit | — | 1,714 |
| MOLE-S | Surface rover | Rim/surface excavation & construction (SPA ONLY) | SPA | 4 | 6 |
| MOLE-I | PSR rover | Shackleton PSR ice extraction (permanent) | SPA | 10 | 355→~50 |
| SINTER | Construction | Regolith sintering, roads, pads, structures | Both | 3 | 200+ |
| SURVEY | Prospector | Geological mapping (surface + orbital) | SPA | 1 | 2 |
| SKIP | Sub-orbital | SPA Basin KREEP (RETIRED P9) | SPA | 1 | 8 |
| DART | Orbital transfer | PKT collector via LLO (pathfinder) | SPA | — | 1 |
| ARM-C / ARM-D | Manipulator | Construction, docking, cargo, maintenance | Both | 2 | 200+ |
| SENTINEL | Inspection | Structural validation, fleet monitoring, safety | Both | 2 | 4,000+ |
| Hauler | Surface vehicle | PKT mining, transport, grading, trenching | PKT | — | 1,488,096 |

★ Key architectural constraints: (1) NO SKIPs AT PKT — EVER. (2) MOLE-S deploys to SPA ONLY. (3) Haulers are PKT-only. (4) PKT is fully autonomous (zero permanent crew). (5) PROBEs base permanently at SPA (propellant constraint). (6) All asteroid processing at SPA, not PKT.

# 2. PROBE — Asteroid Survey & Return Spacecraft

## 2.1 Mk I — Chemical Return Vehicle (P3–P11)

| Parameter | Specification |
| --- | --- |
| Dry mass (Ops) | ~800 kg (body + payload instruments + enlarged tanks) |
| Wet mass (ISRU) | 8,950 kg (800 dry + 1,500 payload + 6,654 propellant) |
| Propellant/mission | 6,654 kg LH₂/LOX (6 km/s round-trip ΔV to low-ΔV NEA) |
| Isp | ~450 s (LH₂/LOX bipropellant); methalox fallback 310 s |
| Payload | 1,500 kg Ni-Fe/PGM semi-raw (no beneficiation) |
| Mission cadence | 1 mission/craft/yr (Ryugu-class ~3–4 km/s one-way from LLO) |
| MOLE-I equivalence | 0.855 PROBE missions per MOLE-I per year (5,692 ÷ 6,654) |
| Docking | PROBE collar: soft-capture + conductive charging pins, SpaceWire data |
| Navigation | Terrain-relative (SLIM heritage, 55 m) + star tracker + IMU + radar alt |
| Fleet | P3: 3 → P5: 10 → P7: 30 → P8: 96 → P9: 30 (phased to Mk II) |

★ Do NOT attempt REE extraction from asteroid payloads. NEA REE ~5 ppm vs 1,000+ ppm economic ore. ★ PROBE-Scout (one-way, 243 kg wet, expendable) for NEA survey — not counted in operational fleet.

## 2.2 Mk II Heavy — Asteroid Processor (P9–P11+)

| Parameter | Specification |
| --- | --- |
| Dry mass | ~5,000 kg (vs 800 kg Mk I) |
| Propellant capacity | 40,000 kg LH₂/LOX |
| In-situ beneficiation | On-asteroid: magnetic drum separator + centrifuge. Processes 500 t, returns 10 t of 50× PGM-concentrated material (~17.5 kg PGM/mission) |
| Dwell time | Weeks to months (vs days for Mk I) |
| Fleet | P9: 190 → P10: 230 → P11: 260 (peak) → phasedown to 30 |

## 2.3 Mk III SEP — Ion Bulk Miner (P11+, DG-11.6)

| Parameter | Specification |
| --- | --- |
| Dry mass | ~1,200 kg. Larger solar arrays (no landing loads, no dust). |
| Propulsion | Ion (xenon/argon). Zero ISRU LH₂/LOX consumption. |
| Landing | NEVER lands on Moon. Stays in orbit permanently. |
| Payload delivery | Ejects 10 t Fe-Ni/PGM canisters from LLO on ballistic deorbit. EM catcher at SPA receives at ~1,680 m/s. |
| Canister | Fe-Ni shell (ferromagnetic for EM braking) + cold-gas kick motor (~5–10 kg) |
| Anchoring | EM anchor pads on M-type Fe-Ni surface. No harpoons/drills. Heritage: MEV-1 (2020). |
| Fleet | Build-out ~86/yr Y60–Y80 → ~1,714 peak → ~300 prospecting (Y115+) |
| Missions | 0.5 missions/yr per unit |

Mk III operational sequence: (1) Ion thrust transit 3–8 months. (2) Velocity matching to cm/s. (3) LiDAR surface mapping at 100 m. (4) Terminal approach at 2–5 cm/s. (5) EM anchor engagement (3–4 pads). (6) Scraper mining 6–12 months. (7) Disengage (cut current), cold-gas separation, ion return.

## 2.4 Fleet Phasedown and Propellant Cascade

| Year | Mk I/II fleet | Mk III fleet | MOLE-I needed | Notes |
| --- | --- | --- | --- | --- |
| Y45 (P10) | 230 | 0 | ~280 | Peak chemical PROBE demand |
| Y60 (P11) | 260 | 0 | ~320 | Mk III build-out begins |
| Y80 | 50 | ~1,714 | ~190 | Mk I/II phasedown near complete |
| Y85 | 30 | ~1,714 | ~50 | Chemical fleet minimal; MOLE-I drops |
| Y115+ | 30 | 1,714→300 | ~50 | All asteroid captures complete |

Decommissioned Mk I/II salvaged at SPA — Fe-Ni frames, motors, electronics recycled (~184 t recovered). Captured asteroids do NOT use PROBEs — dedicated Laser Ablation Truss + mass driver (MD-5, DRO→SPA). Architecturally independent systems.

# 3. MOLE-S — Surface Excavation Rover (SPA Only)

MOLE-S deploys to SPA ONLY. Not deployed to PKT under any circumstance. At PKT, all excavation is performed by mining-capable haulers (SEL-HAULER-001 Rev B).

| Parameter | Specification |
| --- | --- |
| Dry mass | ~350 kg (5× RASSOR 2.0 at 67 kg) |
| Excavation | Dual counterrotating bucket drums (RASSOR heritage, zero net horizontal reaction). 0.45 m dia × 0.50 m. 50 kg/hr. 350 kg per dump. |
| Mobility | 6 independently driven mesh wheels, rocker-bogie, Ti flexure pivots. 0.2 m/s loaded, 0.5 m/s unloaded. 25° grade. |
| Power | 800 W excavation / 250 W transit / 50 W standby. 5,000 Wh Li-ion dual-redundant. Charge: 1,500 W / 120 VDC, ~3.5 hr. |
| Dust mitigation | PTFE labyrinth seals (>1M cycles, NASA tested), EDS optics, WC drum tips. |
| Fleet | 4 (P3) → 6 (P7+). Constant thereafter. Roles: rim construction, SINTER regolith supply, site levelling, pad prep. |

# 4. MOLE-I — PSR Ice Extraction (Demand-Driven)

| Parameter | Specification |
| --- | --- |
| Dry mass | ~360.65 kg (per ECN-013 Rev 2: +NAV-001 IMU, +bayonet adapter) |
| Propellant yield | 5,692 kg/yr per unit (7,818 kg/yr water × 0.722 electrolysis eff.) |
| Mini-VEX | Integrated thermal extraction. ~85% water recovery. 1,200 W operational (sublimation + sensible + regolith heating). Piped to surface via heated pipeline. |
| Power (operating) | ~1,934 W at tether (300 W drive + 200 W drill + 1,200 W mini-VEX + 15 W WEB + 20 W comms + 50 W thermal keep-alive + 99 W DC-DC + 50 W charge) |
| Power (standby) | ~65 W (WEB 15 W + gearbox heaters 30 W + drill bearing 15 W + charge receptacle 5 W) |
| WEB | Warm Electronics Box, 273 K setpoint, Cu braid thermal strap. WEB loss is immediately fatal (electronics freeze at 90 K). LWRHU backup (3×3.3 W). |
| Transit battery | ~334 Wh (1.3 kg). 65 W keep-alive × 4.7 hr × 1.10 margin. 250 Wh/kg Li-ion inside WEB. |
| Navigation | 3-layer: (1) tether polar fix (drum payout + DRM-040 angle encoder), (2) wheel odometry (6×), (3) MEMS IMU (NAV-001). Kalman filter, ±0.5 m. 3,600-cell drill grid. (ECN-013) |
| Bayonet adapter | Motorised (SEL-MI-CON-001), 0.3 kg, 2 W. Active engagement for construction tap connections. Passive detent hold. (ECN-013 Rev 2) |
| Deployment | Rim-anchored winch, Kevlar/Vectran cable (Dyneema excluded: Tg ~123–153 K → brittle). 0.5 m/s, ~4.7 hr over 8,400 m. PSR-permanent. |
| Drain pipe | Rear nozzle (ECN-012). Heated pipeline to surface. Funnel specs unchanged. |
| Fleet | 10→25→50→85→170 (P7) → ~247 (P8) → ~240 (P9) → ~355 peak → ~50 (Y85+) |

Demand-driven correction: Old figures 48,990 (P8) / 390,790 (P9) eliminated. Driven by chemical tanker flights for PKT SKIPs — architecture removed by ECN-019 (mass driver at P8, no SKIPs at PKT). Shackleton floor fits ~310 max. Saving: ~$850B.

## 4.1 Revised Scaling Table

| Phase | Years | MOLE-I | PROBEs | SKIP (SPA) | Supply t/yr | Demand t/yr | Margin | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P3 | Y4–7 | 10 | 3 | 1 | 56.9 | ~0 | Stockpile | Accumulation |
| P4 | Y7–9 | 25 | 6 | 1 | 142 | 124 | +14.7% | Crew arrival |
| P5 | Y9–12 | 50 | 10 | 2 | 285 | 238 | +19.6% | DART begins |
| P6 | Y12–15 | 85 | 15 | 4 | 484 | 434 | +11.5% | Processing online |
| P7 | Y18–25 | 170 | 30 | 8 | 968 | 868 | +11.5% | Peak SKIP fleet |
| P8 | Y25–35 | ~247 | 96 | 8 | 1,406 | 1,403 | +0.2% | Mass driver online |
| P9 | Y35–45 | ~240 | ~190 | 0 | 1,367 | 1,364 | +0.2% | SKIPs retired |
| P10 | Y45–60 | ~280 | ~230 | 0 | 1,594 | 1,630 | −2% | de Gerlache if needed |
| P11 | Y60–80 | ~320→~50 | 260→30 | 0 | var | var | var | Mk III transition |
| P12+ | Y80+ | ~50 | 30 chem | 0 | ~285 | ~300 | C-type | Ion fleet dominates |

Spatial constraints: ~310 positions at Shackleton, ~45 at de Gerlache (5–10 km via Connecting Ridge). At LCROSS 5.6%+ ice, Shackleton alone sufficient.

# 5. SKIP — Sub-Orbital KREEP Hopper (SPA Only, Retired P9)

NO SKIPs AT PKT — EVER.

| Parameter | Specification |
| --- | --- |
| Dry mass | ~300 kg |
| Wet mass (per hop) | ~400 kg (300 dry + ~95 kg propellant + margin) |
| Propellant per hop | 303 kg LH₂/LOX (100 km range, 1,690 m/s round-trip ΔV) |
| Payload | 400 kg KREEP regolith per hop |
| Cadence | 1 hop/day max (limited by pre-excavation rate). 274 hops/yr at 75% avail. |
| Fleet demand (8 units) | ~664 t/yr LH₂/LOX propellant (58 MOLE-I equivalent) |
| Operating range | ~100 km from Shackleton to SPA Basin KREEP exposures |
| Retirement | P9 — PKT 2,500 circuits produce 1,000 t/yr REO; SPA 5-circuit research irrelevant |

# 6. DART — Deep-Range Autonomous Return Transport

| Parameter | Specification |
| --- | --- |
| Transfer architecture | LLO orbital transfer (NOT ballistic arc — 3,300 km requires ~13,200 m/s ΔV ballistically, physically impractical) |
| Round-trip ΔV | ~7,400–8,900 m/s (with 15% margin) |
| Propellant per mission | ~12,370 kg LH₂/LOX at planning ΔV ~8,200 m/s |
| Minimum economic payload | 1,200 kg PKT material (≈ 10,000 kg SPA-equivalent after beneficiation) |
| Mission cadence | Annual/biennial (limited by ISRU stockpile: ~22 months at 5,000 kg/yr to accumulate 9 t) |
| PKT ore grade advantage | 5–10× SPA Basin; 10–15 ppm Th vs 1–3 ppm SPA |
| Fleet | 1 unit. Pathfinder for P5–P6. DART propellant demand zero from P8 (operates on PKT surface power via mass driver). |

# 7. Surface Hauler — Mining-Capable (SEL-HAULER-001 Rev B)

Replaces both SKIP and MOLE-S at PKT. One vehicle class handles all PKT surface operations from P8: mining, transport, grading, trenching, SINTER supply. Full spec: SEL-HAULER-001 Rev B.

| Parameter | Rev A (superseded) | Rev B (ECN-019) |
| --- | --- | --- |
| Dry mass | ~1,200 kg | ~1,450 kg |
| Battery | 200 kg Li-ion, 50 kWh | ~300 kg Na-S, 50 kWh, 300–350°C |
| Motors | 4× SmCo BLDC hub | 4× SRM (Fe-Si laminations, Al windings, no magnets) |
| Mining | None | Front bucket/blade (~150 kg), 200 kg/min, self-load 5 t in ~25 min |
| Introduction | P10 | P8 (Y25) |
| In-situ fraction | 92% | ~57% (Al frame 85%, SRM 70%, Na-S 60%, avionics 0%) |
| Earth frac trajectory | — | 100% (P8) → 43% (P10) → 33% (P12) → 20% floor (post S-type) |

Fleet: 200 (P8) → 1,000+ (P9) → 14,881 (P10) → 148,810 (P11) → 1,488,096 (P12) → ~1,500,000 (P14+ steady state). Charging hubs: 1 per 100 haulers. Na-S thermal management critical — battery must never freeze (<270°C solidification damages cell).

# 8. Steel Conveyor Network (Replaces EM Catapults)

SEL-CATAPULT-001 Rev A SUPERSEDED for ore transport. 9,084 catapult hubs eliminated. Steel apron conveyors (~95% in-situ Fe-Ni + MRE iron) replace Tier 3 transport. YBCO: 209,000 t → 0 for ore transport. Coilgun heritage retained for 5 mass drivers (~280 t YBCO lifetime total).

| Parameter | EM Catapult (superseded) | Steel Conveyor (baseline) |
| --- | --- | --- |
| Annual Earth import (P12) | ~6,140 t/yr (~61 flights) | ~300–430 t/yr (~3–4 flights) |
| YBCO required | ~209,000 t | 0 |
| Continuous power | ~13.6 GW | ~0.44 GW |
| In-situ fraction | ~0% critical | ~95–97% |

Network: dendritic, ~87,500 km at P12. ~500 trunks (75 km avg) + ~5,000 branches (10 km avg). SRM drive stations every 1–2 km. MoS₂ lubricated. Doc required: SEL-CONVEYOR-001 Rev A.

# 9. ARM & SENTINEL — Support Fleet

## 9.1 ARM

| Parameter | Specification |
| --- | --- |
| ARM-D (SPA IZ) | 1:1 ratio with docking collars. Primary arm: 6 m, 7-DOF, 2,000 kg capacity. Dexterous arm: 2.5 m, dual 7-DOF (14 total), ±1 mm. Rail-mounted gantry spans all bays. Grid-powered. Heritage: ESA ERA + CSA Dextre. |
| ARM-C (PKT) | Construction variant. Scales 1 per 500 circuits/yr build rate. Hub station maintenance: wheel swap, battery swap, motor swap. All components ORU. |
| Collar scaling | P3: 2 → P5: 6 → P7: 8 → P11: 30+ (linear with PROBE fleet). SPA IZ gantry rail extendable by design (SEL-CAD-IZ-001). |
| Turnaround | 48–72 hr per PROBE service cycle (dock, ore extract via dust vestibule, refuel, inspect, release). |

## 9.2 SENTINEL

| Parameter | Specification |
| --- | --- |
| Mass | ~80 kg, 4-wheel rover, all-access including beneficiation exterior |
| Roles | Structural validation (pre-crew sign-off gate), ISRU plant health, H₂ leak detection (catalytic + mass-spec), fleet monitoring |
| PKT scaling | 1 per 500 circuits total. At P12 (2.08M circuits): ~4,167 units. |
| SPA complement | 4+ units (unchanged from P6+) |
| ECN-014 | Cryo transfer line monitoring: 4× isolation valve stations, H₂/pressure/UV flame sensors, auto-close logic (hardwired 24 VDC, NOT CAN bus), dead-man fail-closed. O₂ supply line overpressure/high-flow protection. VALVE_STATE DB table (11th table). Fault patterns #201–#206. |

# 10. SINTER — Construction Rover

| Parameter | Specification |
| --- | --- |
| Dry mass | ~2,200 kg (ATHLETE mobility + sintering payload + 10 kWe fission reactor) |
| Sintering | Primary: concentrated solar (2.37 m² Fresnel, 1,000–1,100°C). Secondary: CO₂ laser 5 kW (night ops). Tertiary: 2×1 kW microwave 2.45 GHz (volumetric). |
| Road output | ~100 m/day per unit (Mode A, 3 m width, 100 mm depth) |
| Fleet | 3 (P3) → 20+ (P8) → 200+ (P12) → ~50 (P14+ maintenance). NEVER decommission all units. |
| Arm | 3.0 m reach, 7-DOF, 150 kg payload at tip, quick-disconnect tools |

# 11. SURVEY — Geological Prospector

| Parameter | Specification |
| --- | --- |
| Mass | ~180 kg, 4-wheel rocker-bogie, ~200 W solar + 500 Wh Li-ion |
| Instruments | Ground-Penetrating Radar (subsurface to ~3 m), portable XRF (KREEP boundaries), seismic array (geophone deploy/retrieve) |
| Data output | KREEP grade map → SKIP/DART target AI pipeline |
| Fleet | 2 units (1 surface + 1 orbital). PREREQUISITE: min 20 SKIP target sites before SKIP-1 authorised. |

# 12. Crater Access Infrastructure — PSR Operations

Shackleton Crater PSR: ~4.2 km below rim along ~30° slope (~8.4 km traverse). MOLE-I cannot self-propel up this slope.

| Parameter | Specification |
| --- | --- |
| Descent winch | Rim-anchored, 0.43 kW motorised. Kevlar or Vectran cable ONLY (Dyneema excluded: Tg ~123–153 K → brittle snap-failure risk). Kevlar retains tensile to <4 K (+4% strength, +13% modulus at 77 K vs RT). 0.5 m/s, ~4.7 hr over 8,400 m. One winch per MOLE-I. |
| Cable tramway (P3) | 350 kg containers, 2 m/s, ~19 trips/day (~2,379 t/yr capacity). 0.55 kW lift. Retained as backup after pipeline. |
| Heated pipeline (P4+) | 8,500 m, 7 W/m trace heating (59.5 kW total continuous). Ti-6Al-4V 1 mm wall. 68 bar trunk pump at CLP (408 W, 0.042 kg/s). 34 branch transfer pumps (~10 W each). Redundant parallel pipe from P5. MUST NEVER LOSE POWER — freeze = total replacement. |
| PSR charging network | Bilateral fishbone: trunk cable (1,000 VDC OFHC Cu, 304 mm²) + branches elevated ~3 m on sintered stakes. 62 junctions support up to 620 MOLE-I; P7+ uses 34 nodes (170 units). Each node: 5 MOLE-I + 1 spare, WEB (80 W), 5 cable drums. Branch pipes slope ≤2° toward trunk (gravity-fed). |
| SATS+WEB assembly | Pre-integrated (ECN-013 Rev 2). SATS frame ~10 kg + WEB ~15 kg = ~25 kg. Pre-heated to 293 K, lowered at trunk end. 3× LWRHU (3.3 W) maintain WEB ≥268 K during 23-hr descent. Arrives powered and operational. No PSR-floor assembly. |
| CAP (contingency) | Pressurised crew pod, ~850 kg dry, ~1,400 kg loaded (2 crew + 2 EVA suits). 6 m³, 18 person-hr life support. Dedicated crew-rated winch. Phase 2+ reservations only (no schedule impact). Does NOT solve floor EVA thermal limits (2 hr at 40 K). |

# 13. Processing Circuit Design

| Design | REO/yr | Mass (kg) | Power (kW) | Circuits for 2.5 Mt/yr | Total mass |
| --- | --- | --- | --- | --- | --- |
| Batch (rejected) | 0.4 t | 15,000 | 180 | 6,250,000 | 93.8 Mt |
| Continuous-flow | 0.8 t | 22,000 | 300 | 3,125,000 | 68.8 Mt |
| Optimised (committed) | 1.2 t | 28,000 | 400 | 2,083,334 | 58.3 Mt |

Highest-value R&D investment ($22T cost difference batch vs optimised). Maintenance: 0.5% mass/yr. Earth frac: 100% (P8) → 10% (P10+) → 3% (post C-type) → 0.8% (post S-type). Gate DG-8.X: validate ≥1.0 t/yr before PKT commitment.

# 14. Construction Fleet Scaling

| Vehicle | Scales with | Mass | Replace | Notes |
| --- | --- | --- | --- | --- |
| ARM-C | 1 per 500 circuits/yr build rate | 1,500 kg | 10-yr | Circuit installation at PKT |
| SINTER | 1 per 200 circuits/yr build rate | 2,200 kg | 15-yr | Roads, pads, enclosures |
| SENTINEL | 1 per 500 circuits total | 80 kg | 10-yr | Monitoring + safety |
| Charging hub | 1 per 100 haulers | 5,000 kg | Long | 95% in-situ; Na-S thermal mgmt |

Steady-state minimum (P14+): ~20 ARM-C, ~50 SINTER, SENTINEL scales with circuit count. Construction fleet cargo: ~0.3% of total Earth cargo.

# 15. Asteroid Capture Infrastructure

| # | Type | Size | Capture | Provides | Impact |
| --- | --- | --- | --- | --- | --- |
| 1 | M-type | ~200 m | Y75→Y85 | 36.5 t/yr PGM, Fe-Ni for foundry | 80+ yr campaign; hauler Earth frac →20% |
| 2 | C-type | ~100 m | Y95→Y105 | Water, carbon, nitrogen | Circuit frac 10%→3%; propellant indep.; PTFE in-situ |
| 3 | S-type | ~100 m | Y115→Y125 | Silicon, copper | Circuit frac 3%→0.8%; sensors/wiring in-situ |

Redirect tugs: ~50,000 kg each, SEP/NEP, 20-yr life, Earth-built at $500/kg LEO. Fleet: 2→3→5. Safety: planetary defence review (UN COPUOS), impact probability <10⁻⁸, multiple abort windows.

Laser Ablation Truss: 1 per captured asteroid in DRO. Linear rigid truss, dual laser heads, ~350 MW, 1,000 t/day. Surface ablation spiral pattern (no diameter cuts). Electrostatic/magnetic collectors. Heritage: DE-STAR (Lubin, UCSB). DG-13.2.

All processing at SPA: M-type PGM hydromet, C-type pyrolysis + Fischer-Tropsch, S-type Czochralski Si + Cu electrolysis. Products mass-driven SPA→PKT.

# 16. SPA Industrial Zone — Docking Architecture

Docking collars are horizontal cylinders (5 m OD × 4.5 m length, axis along X) in a single linear row. ARM-D ratio: 1:1 per collar. Collars scale linearly with PROBE fleet: P3: 2 → P5: 6 → P7: 8 → P11: 30+. Gantry rail extendable by design (SEL-CAD-IZ-001). At 286 PROBEs with ~1 mission/yr and 48–72 hr turnaround, 30 collars give ~8% utilisation.

Mk III PROBEs never dock at IZ (never land). Their canisters arrive via EM catcher (separate from docking gantry). SPA IZ infrastructure handles Mk I/II only.

# 17. Document History

| Version | Description |
| --- | --- |
| v8 | 17×5 architecture, separated ISRU, 9 robot classes, MOLE split, SKIP/DART/SURVEY added. March 2026. |
| ECN-012 | MOLE-I drain pipe reroute to rear nozzle. Heated pipeline integration. Power budget note. |
| ECN-013 | MOLE-I 3-layer navigation (tether polar fix + odometry + IMU). ARM removed from PSR deployment. SATS+WEB pre-integrated assembly. Motorised bayonet adapter (SEL-MI-CON-001). Construction tether retraction spools. |
| ECN-013 R2/R3 | SATS+WEB descent procedure formalised. Bayonet adapter spec (0.3 kg, 2 W, detent hold). Spring-loaded spool auto-retract (1.0 kg/spool, 5 N constant tension). |
| ECN-014 | SENTINEL cryo transfer line monitoring. Auto-close protocol (hardwired 24 VDC). O₂ supply monitoring. VALVE_STATE DB table. Fault patterns #201–#206. |
| ECN-016 | SKIP phase-out schedule, MOLE-I Mk II deep drill retrofit, PROBE Mk II heavy, hauler introduction, EM catapult, asteroid capture strategy. (Partially superseded by ECN-019.) |
| ECN-017 | In-situ hauler at 92%, corrected catapult scaling. (Superseded by ECN-019.) |
| v9 | Programme-wide revision per ECN-019 Rev C. 2.5 Mt/yr target. Demand-driven MOLE-I (max ~355, drops to ~50). SKIP retired P9 — NO PKT SKIPs. PROBE Mk III SEP. Hauler Rev B at P8 (Na-S, SRM, mining bucket, ~57% in-situ). Catapults → conveyors. Optimised circuits (1.2 t/yr, 2.08M). Base specialisation (SPA=asteroid, PKT=KREEP). Diversified asteroids (M/C/S). Laser ablation truss. 5 mass drivers (~280 t YBCO lifetime). Construction fleet scaling. P13/P14/P14+ phases. Earth independence trajectory (circuit frac 0.8% by Y135). April 2026. |

# APPENDIX A — Operational Vehicle Fleet Counts (All Phases)

End-of-phase counts. Mk III uses zero ISRU propellant. Haulers PKT-only. SKIPs/MOLE-S SPA-only.

| Vehicle | P0–P3 | P4 | P5 | P6 | P7 | P8 | P9 | P10 | P11 | P12 | P13 | P14 | P14+ | Base |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PROBE Mk I | 3 | 6 | 10 | 15 | 30 | 96 | 30 | 30 | →0 | — | — | — | — | SPA |
| PROBE Mk II | — | — | — | — | — | — | 190 | 230 | 260→30 | 30 | 30 | 30 | 30 | SPA |
| PROBE Mk III | — | — | — | — | — | — | — | — | 0→1,714 | 1,714 | 1,714 | 1,714→300 | 300 | Orbit |
| SKIP (SPA) | 1 | 1 | 2 | 4 | 8 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | SPA |
| MOLE-I | 10 | 25 | 50 | 85 | 170 | ~247 | ~240 | ~280 | ~320→~50 | ~50 | ~50 | ~50 | ~50 | SPA |
| MOLE-S | 4 | 4 | 4 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | SPA |
| DART | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | SPA |
| SURVEY | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | SPA |
| Hauler | 0 | 0 | 0 | 0 | 0 | 200 | 1k+ | 14,881 | 148,810 | 1.49M | ~1.5M | ~1.5M | ~1.5M | PKT |
| Redirect tug | — | — | — | — | — | — | — | — | 2 | 3 | 3–5 | 5 | 5 | DRO |
| Laser truss | — | — | — | — | — | — | — | — | — | 1 | 1–2 | 2–3 | 3 | DRO |

# APPENDIX B — Infrastructure & Construction Fleet (All Phases)

| System | P0–P3 | P4 | P5 | P6 | P7 | P8 | P9 | P10 | P11 | P12 | P13 | P14 | P14+ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SINTER | 3 | 3 | 3 | 3 | 3+ | 20+ | 30+ | 50+ | 100+ | 200+ | 200+ | 100+ | ~50 |
| ARM-C (PKT) | 0 | 0 | 0 | 0 | 0 | 5+ | 15+ | 30+ | 100+ | 200+ | 150+ | 100+ | ~20 |
| ARM-D (SPA) | 2 | 4 | 6 | 8 | 8 | 10 | 15 | 20 | 30+ | 30+ | 30+ | 30+ | 30+ |
| SENTINEL SPA | 2 | 2 | 3 | 4 | 4+ | 6+ | 10+ | 15+ | 20+ | 20+ | 20+ | 20+ | 20+ |
| SENTINEL PKT | 0 | 0 | 0 | 0 | 0 | 5+ | 10+ | 50+ | 500+ | 4,167+ | 4,167+ | 4,167+ | maint |
| Charging hub | 0 | 0 | 0 | 0 | 0 | 2 | 10 | ~150 | ~1,500 | ~15,000 | ~15,000 | ~15,000 | ~15,000 |
| Conveyor km | 0 | 0 | 0 | 0 | 0 | 0 | 500 | 5k+ | 15,000 | 87,500 | 87,500 | 87,500 | 87,500 |
| Road km | ~1 | ~3 | ~10 | ~30 | ~100 | ~500 | ~2k | ~15k | ~50k | ~200k | ~200k | ~200k | ~200k |
| Mass drivers | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 | 5 | 5 | 5 | 5 |
| Circuits | 0 | 0 | 0 | 0 | 5 | 308 | 2,500 | 25k | 250k | 2.08M | 2.08M | 2.08M | 2.08M |
| MSR (100MW) | 0 | 0 | 0 | 0 | 0 | 0 | 1–3 | ~74 | ~679 | ~6,890 | ~6,890 | ~6,893 | ~6,893 |
| REO t/yr | 0 | 0 | 0 | 0 | 0.4 | 125 | 1,000 | 10k | 100k | 500k | 1.25M | 2.5M | 2.5M |

# APPENDIX C — Phase Timeline and Key Milestones

| Phase | Years | Calendar | REO t/yr | Key milestones |
| --- | --- | --- | --- | --- |
| P0 | Y0–3 | ~2025–28 | 0 | Programme GO, relay deployment, robot fleet design freeze, FSP spec lock |
| P1–3 | Y2–7 | ~2027–32 | 0 | Cargo landing, ISRU commissioning, MOLE-I deployment, SINTER construction, first ice extraction |
| P4 | Y7–9 | ~2032–34 | 0 | Crew arrival (4–8), life support online, PROBE fleet ISRU-fuelled |
| P5 | Y9–12 | ~2034–37 | 0 | DART first PKT mission (M127), beneficiation, troilite H₂SO₄ pilot (M115), first export (M116) |
| P6 | Y12–15 | ~2037–40 | 0 | Processing facility, Ca(OH)₂ pilot, DART PKT missions 2–3, crew rotation begins |
| P7 | Y18–25 | ~2043–50 | 0.4 | PKT commitment (DG-7.1), Th recovery begins (DG-7.2), 5 circuits, 8 SKIPs (peak), YBCO delivery for MD-1 |
| P8 | Y25–35 | ~2050–60 | 125 | SPA→PKT mass driver (DG-8.0), PKT 4-wave construction (~300 Starships), 308 circuits, circuit validation (DG-8.X), haulers at PKT, catapult demo |
| P9 | Y35–45 | ~2060–70 | 1,000 | PKT→Earth mass driver (DG-9.1), SKIP retired, first MSR (DG-9.3), foundry scale, conveyor pilot, 2,500 circuits |
| P10 | Y45–60 | ~2070–85 | 10,000 | 14,881 haulers, 25k circuits, MSR fleet ~74 (DG-10.4), SPA MSR (DG-10.5), full industrial scale |
| P11 | Y60–80 | ~2085–2105 | 100,000 | Mk III PROBE (DG-11.6), MOLE-I →50, M-type redirect (Y75), SPA→Earth mass driver, Ni-201 MSR vessels |
| P12 | Y80–120 | ~2105–45 | 100k→500k | M-type online (Y85), 36.5 t/yr PGM, laser truss, MD-5, 1.49M haulers, C-type redirect (Y95) |
| P13 | Y120–140 | ~2145–65 | 500k→1.25M | C-type online (Y105), SPA MSR #1, propellant independence, circuit frac →3%, S-type redirect (Y115) |
| P14 | Y140–180 | ~2165–2205 | 1.25M→2.5M | S-type online (Y125), Si/Cu in-situ, SPA MSR #2/#3, circuit frac →0.8%, Earth independence (DG-14.6) |
| P14+ | Y180+ | ~2205+ | 2,500,000 | Steady state. +$121.8B/yr surplus. 182 Mt CO₂/yr avoided. <0.5% Earth mass dependency. Indefinite operation. |

— End of SEL_ROBOT_FLEET v9 —